LAB LOG

Two SO Arms — Leader and Follower

Part A is the first arm (unchanged notes). Part B is only the files changed for the second arm.

| Field | Detail |
| --- | --- |
| Leader (arm 1) | Mostly COM7 bring-up, later COM5 for teach / follow |
| Follower (arm 2 / Robo 2) | COM5 in the new files. Own limits, rest, and pose JSON |
| Motors | ST3215 / HS3215, six joints each |
| Library | python_st3215 |

# Part A — First arm (kept)

Tests 1–4 notes are still: code first, then wiring, then the power supply was off, then IDs 1–6 read but write_goal_position failed.

FINAL tools from the first arm: disable-torque.py, rest_pos_with_s-curve.py (old rest 2048 / 759 / 3054 / 1926 / 3385 / 2301), new_save-custom_pos.py, record_motions.py, move_recorded_motion.py, collaborate_both-robots.py, hand control robot.py.

Unchanged copies are not listed again below.

# Part B — Second arm only

These files were copied and then edited so the follower has its own IDs, limits, rest, and pose book. COM port in this set is COM5 unless a line says otherwise.

| Test | File | What changed |
| --- | --- | --- |
| B1 | assign-new-ids.py | NEW — rewrite servo EEPROM IDs |
| B2 | diagnostic-id3.py | NEW — full status dump for ID 3 |
| B3 | min-max-pos-mimic.txt | NEW — measured travel, including wrap joints |
| B4 | middle-position.txt | NEW — hand-saved mid pose |
| B5 | rest-position.txt + rest_pos_with_s-curve.py | New rest numbers for this arm |
| B6 | move-robo2-to-middle-pos-new-values.py | NEW — mid pose with wrap check |
| B7 | move to center with deceleration formula.py | Now uses per-joint mids, not 2048 |
| B8 | save_position_custom1.py | Writes recorded_poses_robo2.json |
| B9 | move to x pose2.py | Reads recorded_poses_robo2.json |
| B10 | move to x pose_new.py | Clamps every target to Robo 2 limits |
| B11 | halfpos_trial.py | Same rest move, port now COM5 |

## Test B1  —  assign-new-ids.py

File:  assign-new-ids.py      Result:  NEW  —  change stored servo IDs

### What the code does

- Opens COM5 and scans IDs 0–20.

- Needs exactly six live IDs. Parks them on 200–205 first so nothing collides.

- Then writes each parked ID onto TARGET_IDS. In this file TARGET_IDS is 1–6.

- Each change: torque off, unlock EEPROM, eeprom.write_id(), lock, read the new ID back.

The comment at the top says 7–12. The lists in the file send the motors to 1–6. Use one bus with one motor if every factory ID is still 1.

What this means

## Test B2  —  diagnostic-id3.py

File:  diagnostic-id3.py      Result:  NEW  —  why ID 3 stalls or wraps

### What the code does

- COM5, ID 3 only. Turns torque off first.

- Prints position, load, temperature, voltage, current, EEPROM min/max, and the status byte.

Use this when a joint is hot, stuck at a stop, or the EEPROM angle window is too tight.

What this means

## Test B3  —  min-max-pos-mimic.txt

File:  min-max-pos-mimic.txt      Result:  measured travel for the follower

Joints 3 and 5 wrap past 4095, so (min + max) / 2 is not the middle of the real arc. The file lists both the raw average and a wrap mid.

| ID | Joint | Min | Max | Use this mid |
| --- | --- | --- | --- | --- |
| 1 | Base | 752 | 3443 | 2097 |
| 2 | Shoulder | 804 | 3209 | 2006 |
| 3 | Elbow (wrap) | 1856 | 28 | 2990 |
| 4 | Wrist Pitch | 843 | 2911 | 1877 |
| 5 | Wrist Roll (wrap) | 3752 | 218 | 4033 or 1985 |
| 6 | Wrist Yaw | 717 | 2155 | 1436 |

Two later scripts pick different mids for joint 5 (4033 vs 1985). Both are on the wrap circle. Record which one the arm actually used.

What this means

## Test B4  —  middle-position.txt

File:  middle-position.txt      Result:  hand-saved mid snapshot

| ID | Counts | Angle |
| --- | --- | --- |
| 1 | 2088 | 183.52° |
| 2 | 2009 | 176.54° |
| 3 | 930 | 81.74° |
| 4 | 1870 | 164.36° |
| 5 | 3880 | 341.02° |
| 6 | 1431 | 125.77° |

This mid for joint 3 (930) is the simple average, not the wrap mid 2990. Different files use different “middle” numbers.

What this means

## Test B5  —  rest for Robo 2

File:  rest-position.txt  and  rest_pos_with_s-curve.py      Result:  rest numbers changed

### What changed

- Port is COM5.

- New rest in rest-position.txt / rest_pos_with_s-curve.py:

| ID | Counts | Angle |
| --- | --- | --- |
| 1 | 2085.5 | 183.30° |
| 2 | 2014 | 177.06° |
| 3 | 2995 | 263.23° |
| 4 | 2024 | 177.89° |
| 5 | 3885 | 341.50° |
| 6 | 1429 | 125.60° |

Robo 2 teach script uses a different rest again: 2047, 829, 2990, 2906, 2907, 713 (see B8).

What this means

## Test B6  —  move-robo2-to-middle-pos-new-values.py

File:  move-robo2-to-middle-pos-new-values.py      Result:  NEW  —  wrap-aware mid move

- COM5. Targets 2097, 2006, 2990, 1877, 1985, 1436.

- in_allowed_range() treats min > max as a wrap (min…4095 and 0…max).

- Prints OK or OUT OF RANGE before the move, then the error after.

What this means

## Test B7  —  move to center with deceleration formula.py

File:  move to center with deceleration formula.py      Result:  better than all-2048

### What changed vs the first-arm version

- COM5. Each joint has its own target from min-max-pos-mimic.txt (2097, 2006, 2990, 1877, 4033, 1436).

- Still uses accel register 41 and speed 46, sync write, wait until still.

- Prints target vs actual after the move.

What this means

## Test B8  —  save_position_custom1.py  (Robo 2)

File:  save_position_custom1.py      Result:  FINAL for Robo 2 teach

### What changed

- Save file is recorded_poses_robo2.json, not recorded_poses.json.

- Rest baked into this copy: 2047, 829, 2990, 2906, 2907, 713.

- Prompt is teach_robo2>. Same commands: name, l, list, q.

What this means

## Test B9  —  move to x pose2.py

File:  move to x pose2.py      Result:  FINAL play for Robo 2 JSON

- Same as move_to_x_pos1.py, but loads recorded_poses_robo2.json on COM5.

What this means

## Test B10  —  move to x pose_new.py

File:  move to x pose_new.py      Result:  adds limit clamp + built-in MIDPOINT

### What is better than pose2

- Every target is clipped to SERVO_LIMITS before it is sent.

- Always offers a MIDPOINT pose even if the JSON is missing.

| ID | Clamp min | Clamp max | Mid used |
| --- | --- | --- | --- |
| 1 | 726 | 3451 | 2089 |
| 2 | 813 | 3205 | 2009 |
| 3 | 1 | 1860 | 931 |
| 4 | 846 | 2894 | 1870 |
| 5 | 3752 | 4008 | 3880 |
| 6 | 707 | 2156 | 1432 |

These clamps do not model wrap the same way as min-max-pos-mimic.txt. Joint 3 here is 1–1860, not 1856→28.

What this means

## Test B11  —  halfpos_trial.py

File:  halfpos_trial.py      Result:  port changed to COM5

- Same first-arm rest dict (2048, 759, 3054…). Only the port moved. Prefer B5 / B6 for Robo 2 numbers.

What this means

## Robo 2 pose book

File:  recorded_poses_robo2.json      Result:  follower saved poses

Rest block: 2047, 829, 2990, 2906, 2907, 713. Named poses include STAND_UP, REST_POSITION, REST_POS_FINAL, REST. Some early saves only stored two joints.

poses.json is an older COM5 snapshot (REST_POSITION, REST1). Keep it as history, not as the live file.

# Numbers that must not be mixed

| Set | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Arm 1 FINAL rest | 2048 | 759 | 3054 | 1926 | 3385 | 2301 |
| Robo 2 rest.txt | 2086 | 2014 | 2995 | 2024 | 3885 | 1429 |
| Robo 2 teach rest | 2047 | 829 | 2990 | 2906 | 2907 | 713 |
| Wrap mid (mimic txt) | 2097 | 2006 | 2990 | 1877 | 4033 | 1436 |
| middle-position.txt | 2088 | 2009 | 930 | 1870 | 3880 | 1431 |

Arm 1 and Robo 2 do not share rest or mid. Pick one table per arm and keep JSON files separate.

# EEPROM ID commands (from B1)

servo.sram.unlock() servo.eeprom.write_id(new_id) new_servo = controller.wrap_servo(new_id) new_servo.sram.lock() new_servo.eeprom.read_id()

LeRobot Setup and Teleoperation

Follower Calibration

lerobot-calibrate \

--robot.type=so101_follower \

--robot.port=/dev/ttyACM0 \

--robot.id=my_follower_arm

Leader Calibration

lerobot-calibrate \

--teleop.type=so101_leader \

--teleop.port=/dev/ttyACM0 \

--teleop.id=my_follower_arm

Teleoperation

The leader arm was manually controlled to make the follower arm reproduce its movements.

lerobot-teleoperate \

--robot.type=so101_follower \

--robot.port=/dev/ttyACM0 \

--robot.id=my_follower_arm \

--teleop.type=so101_leader \

--teleop.port=/dev/ttyACM0 \

--teleop.id=my_follower_arm

Dataset Recording

A dataset was recorded by manually controlling the leader arm while the follower arm copied its movements. The task was to pick up a red cube and place it inside a box.

A negative dataset was also recorded. In this dataset, the cube was removed from the environment and the robot was not moved. This provided examples of the robot remaining stationary when the target object was absent.

ACT Policy

Learnt about the ACT (Action Chunking with Transformers) policy, including the role of weights and biases in the learned model. The recorded demonstrations are used to train the policy to predict the actions required for the follower robot to perform the task.